第二周学习汇报

最终代码：/* USER CODE BEGIN Header */

/**

******************************************************************************

* File Name          : freertos.c

* Description        : 基于 FreeRTOS 的多任务协作 (LED/Key/Uart)

* Board              : 江科大 STM32F103C8T6 (LED:PC13, Key:PA0)

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "FreeRTOS.h"

#include "task.h"

#include "main.h"

#include "cmsis_os.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

#include <stdio.h>

#include <string.h>

/* USER CODE END Includes */

/* Private variables ---------------------------------------------------------*/

/* USER CODE BEGIN Variables */

extern UART_HandleTypeDef huart1;

uint32_t g_led_delay = 500;

osThreadId_t LED_TaskHandle;

const osThreadAttr_t LED_Task_attributes = {

.name = "LED_Task",

.stack_size = 128 * 4,

.priority = (osPriority_t) osPriorityNormal,

};

osThreadId_t Key_TaskHandle;

const osThreadAttr_t Key_Task_attributes = {

.name = "Key_Task",

.stack_size = 128 * 4,

.priority = (osPriority_t) osPriorityNormal,

};

osThreadId_t Uart_TaskHandle;

const osThreadAttr_t Uart_Task_attributes = {

.name = "Uart_Task",

.stack_size = 256 * 4,

.priority = (osPriority_t) osPriorityBelowNormal,

};

osMessageQueueId_t MsgQuenueHandle;

const osMessageQueueAttr_t MsgQuenue_attributes = {

.name = "MsgQuenue"

};

/* USER CODE END Variables */

/* Function Prototypes -------------------------------------------------------*/

void StartLEDTask(void *argument);

void StartKeyTask(void *argument);

void StartUartTask(void *argument);

void MX_FREERTOS_Init(void);

/**

* @brief  FreeRTOS

*/

void MX_FREERTOS_Init(void) {

MsgQuenueHandle = osMessageQueueNew (1, sizeof(uint32_t), &MsgQuenue_attributes);

LED_TaskHandle = osThreadNew(StartLEDTask, NULL, &LED_Task_attributes);

Key_TaskHandle = osThreadNew(StartKeyTask, NULL, &Key_Task_attributes);

Uart_TaskHandle = osThreadNew(StartUartTask, NULL, &Uart_Task_attributes);

}

/* ---------------------------------------------------------------------------*/

/* ---------------------------------------------------------------------------*/

void StartLEDTask(void *argument)

{

uint32_t local_delay;

for(;;)

{

taskENTER_CRITICAL();

local_delay = g_led_delay;

taskEXIT_CRITICAL();

HAL_GPIO_TogglePin(GPIOC, GPIO_PIN_13); // 翻转江科大板载绿灯

osDelay(local_delay);

}

}

/* ---------------------------------------------------------------------------*/

/* ---------------------------------------------------------------------------*/

void StartKeyTask(void *argument)

{

uint32_t freqs[] = {200, 500, 1000};

uint8_t idx = 1;

GPIO_PinState last_state = GPIO_PIN_RESET;

for(;;)

{

GPIO_PinState current_state = HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0);

if(last_state == GPIO_PIN_RESET && current_state == GPIO_PIN_SET)

{

osDelay(20);

if(HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0) == GPIO_PIN_SET)

{

idx = (idx + 1) % 3;

uint32_t new_val = freqs[idx];

taskENTER_CRITICAL();

g_led_delay = new_val;

taskEXIT_CRITICAL();

osMessageQueuePut(MsgQuenueHandle, &new_val, 0, 0);

}

}

last_state = current_state;

osDelay(20);

}

/* ---------------------------------------------------------------------------*/

/* ---------------------------------------------------------------------------*/

void StartUartTask(void *argument)

{

char msg[100];

uint32_t current_val = 500;

uint32_t rx_val;

uint32_t last_print_tick = osKernelGetTickCount();

for(;;)

{

if (osMessageQueueGet(MsgQuenueHandle, &rx_val, NULL, 10) == osOK)

{

current_val = rx_val;

sprintf(msg, "\r\n[Event] Key Pressed! New Freq: %ld ms\r\n", current_val);

HAL_UART_Transmit(&huart1, (uint8_t*)msg, strlen(msg), 10);

}

if ((osKernelGetTickCount() - last_print_tick) >= 1000)

{

sprintf(msg, "Status: System Running... Current LED Interval: %ld ms\r\n", current_val);

HAL_UART_Transmit(&huart1, (uint8_t*)msg, strlen(msg), 10);

last_print_tick = osKernelGetTickCount();

}

osDelay(10);

}

代码分析：

- 使用了全局共享变量+临界区保护

在文件开头定义了：uint32_tg_led_delay=500;

在KEY任务代码中加上：

taskENTER_CRITICAL();//开启保护；不准切换任务，不准中断
       g_led_delay = new_val;//修改核心参数
       taskEXIT_CRITICAL();//结束保护

作用：保护数据一致性；因LED任务需要高频获取变量，所以让其直接读取全局变量更方便

代码修改过程：

- 问题：一开始跟着网上参考用的按键触发是：GPIO_PIN_RESET（低电平按下），但烧录后无法实现按下换挡位的功能，后面查询得知，我使用的江科大开发板的按键电路通常是一段接PA0,另一端接3.3V。这意味着按下按键时，PA0接收到的是高电平（set），这样就导致了程序一直在等低电平，我就算按下按键也不会有反应。

解决方法：改成了适配我的开发板的按键按下后为高电平的特性的GPIO_PIN_SET.

心得：

通过这次代码问题的发现与解决，我明白了要写出适配的任务代码就要仔细了解自己所用的开发板的特性特点，不要盲目找网上的参考写代码。